# Safety Rules for Production

General Safety Practices

- If you are injured or become ill on the job, immediately inform your supervisor

- If you see another employee get hurt or become ill do not attempt to move him/her unless failure to do so would result in further injury or danger.  Call your supervisor.  Only employees trained in First Aid should render assistance

- No employee shall go for medical treatment of any on-the-job injury without an authorization from Human Resources or the Production Manager.

- When you see a co-worker doing something unsafe, point it out to him/her.  If he/she continues to do it, notify your Supervisor.

- Cooperation in accident investigation is required by all employees.

- If you are aware of any defects or dangerous conditions in the equipment or machinery that might cause injury, inform your supervisor immediately.

- Portable electrical equipment and appliances shall be securely placed and guarded to avoid injury to anyone.  Regard all loose wires, broken connections, etc. as "HOT".  Report these immediately.

- All work areas are to be kept clean and orderly.  All trash must be disposed of in the proper containers.

- To avoid accident caused by litter or spills pick it up or wipe it up.

- Know the type of liquids you may have to handle in your work.  Protect yourself in the manner instructed by your supervisor when handling toxic liquids, flammable materials or other liquids.

- There should always be at least one employee on each shift trained in First Aid to handle emergency medical treatment. He/She will be familiar with procedure in the event of a severe accident.

- Good housekeeping skills require that you clean your work station thoroughly before you go on to another assignment.

- Horseplay, unnecessary boisterous conduct, provoking or engaging in any disturbance is dangerous and is strictly forbidden.

- Possession, consumption or sale on Company property of alcohol or any controlled substance is strictly prohibited.

- Do not use a chair, boxes, desks, etc. in place of a ladder or step stool.

- The drawers of desks and file cabinets shall be closed when left unattended, and no more than one file drawer should be opened at any time.

- File cabinets should be properly loaded from bottom drawer up to prevent tipping.

- In the event of fire, sound alarm and evacuate.

- Upon hearing fire alarm, stop work and proceed to the nearest clear exit.  Gather at the designated location.

- Only trained workers may attempt to respond to a fire or other emergency.

- Exit doors must comply with fire safety regulations during business hours.

Machinery and Equipment

- Do not operate any powered tool, appliance, or machine until you have received training from a Supervisor and are thoroughly familiar with its total operation.

- Only operate equipment if you have been given permission to do so by your Supervisor.

- If something goes wrong with equipment or a machine that you are operating, turn it off immediately and notify your Supervisor.

- Never remove or alter safety warnings or instruction posters on or near machinery.

- Use only hand tools that are in good condition, and know the proper use of tools before using them.

- Before repairing any power equipment or machines, the person assigned to the job must:

a.	Shut off the equipment by pulling the proper disconnect switch and applying a lockout tag to the switch.

b	Try the start switch to be sure the proper switch has been disconnected.

- Keep your hands and arms away from moving parts of machinery and out from under or between dies, punches, shear blades, or any other point of machine operation.

- Always wear safety glasses when using compressed air.  Never use air hoses to remove dust from clothing.  Exercise extreme care to protect co-workers in the vicinity.

- Concentrate on your job.  Do not talk with others while operating a machine.  Attention may be lost and this can result in the injury and/or death.

- Never leave tools where they may fall or become a tripping hazard.

- Metal ladders must not be used when working on or near electric lines or equipment.

- A defective ladder should be tagged and removed from use.

- No extension ladder may be used unless it is equipped with safety feet and secured so it will not slip.

- Never remove safety guards from any machinery unless it is necessary to perform repair work.  When the repair work is completed, immediately replace the guards in their proper position.

- All electrical panels and switch boxes shall be identified with a description of the equipment they control, such as: "Proper Labeling of Circuit Breakers"

- Do not use oil, grease, or any other combustible material around or on oxygen equipment.  It may result in an explosion.

- Machinery, equipment and the surrounding area shall be kept free of excessive oil and grease and all other debris.

- Never interfere with the use of any method or process adopted for the protection of all employees.

Protective Equipment

- Approved NIOSH/MSHA respirators shall be worn at all times while painting.

- Respirators, compressors and hoses shall be regularly inspected through a preventative maintenance program.

- Certain jobs are designated as requiring ear protection.  When performing these jobs, protective devices must be used.  Protective devices are available to anyone who wishes to use them even though they are not working in a high noise area.

- Only clothing appropriate to the nature of the job may be worn.  No shorts are allowed anywhere in the facility.

- Welders and spray painters are required to wear full length sleeves, as well as other special protective apparel that is special to their occupation.

- Closed, leather work shoes are required in all areas of the plant.  Employees working in the office who frequently go into plant areas shall also comply with this regulation.  Athletic-type shoes are not allowed.

- OSHA approved safety glasses shall be worn in all areas of manufacturing at all times.

- All plant employees with hair below the collar line must wear their hair up under a head covering or tied back in a "pony tail".

- When walking through the plant, it is strictly prohibited that short-cuts be taken.  It is mandatory that everyone walk through the proper doors and use the proper walkways.

- Back supports are available to employees upon request.

Material Handling

- When lifting, remember that your greatest source of strength is in your legs.  So always bend the knees and lift with your legs.  This is the secret to protecting yourself from back injury or hernia while lifting objects.

- Do not try to lift or push excessive loads by yourself.  Use the "buddy" system.  Get help.

- Stack material safely.  Use proper pallets and blockings.  Never exceed a safe height or block aisles, exits, fire fighting equipment, electrical power panels, sprinklers etc.

- Do not use defective equipment.  (e.g., slings, hooks, chains, etc.)  Report any problems with equipment to your supervisor immediately.

- If the load being carried on a forklift obstructs the operators view, the driver is required to operate the fork lift in reverse.

- When operating overhead cranes, observe the rated capacity of the crane.  Do not attempt to lift a load that exceeds the stated weight.

- When transporting parts, be sure the load is secure to prevent shifting in transit.  Also transport the load as low to the ground as possible.

- Never walk beneath a suspended load when a crane is in operation.

- Before using any type of lift vehicle, be alert to any irregularities that may tell you the vehicle may not perform safely (e.g., cracked forks.)

- When stacking parts on shelves, arrange them in such a manner that prevents the parts from falling off the shelf.

- Material must be stacked where it is not a hazard and where it will not impede safety devices, fire fighting equipment or block aisles, hallways, and exits.

- When using wooden pallets, be sure the pallet is in good condition and suitable to hold the load.  All unsafe pallets should be repaired or discarded.

- When handling or storing compressed air cylinders observe the following regulations:

a.	The valve shall be closed and the cap securely affixed in place on the disconnected cylinders.

- Cylinders shall be secured in an upright position by chain or other means.

- Movement of cylinders from place to place shall be done with the use of the provided cart.

- All cylinders shall be marked as to contents.  Use the correct gas for each        application.  Any cylinder not marked should be reported to the Supervisor.

Vehicle Operating Rules

- Only drivers authorized by the Company and instructed in the safe operation of vehicles shall be permitted to operate such vehicles.

- Stunt driving and horseplay are prohibited.

- No one other than the operator may ride on forklifts, trucks or other vehicles unless adequate riding facilities are provided.

- Employees shall not ride on the forks of forklifts.

- Employees shall not place any part of their bodies outside the running lines of a vehicle or between the masts, uprights or other parts of a forklift where shear or crushing hazards exist.

- Employees shall not be allowed to stand, pass or work under the elevated portion of any forklift, loaded or empty, unless it is effectively blocked to prevent it from falling.

- Drivers shall check Company vehicles at least once per shift.  As a minimum, a safety check of a vehicle shall include checking tires, horn, lights, batteries, controllers, brakes, steering mechanism, and the lift system of forklifts (forks, chains, cables, and limit switches).

If a vehicle is found to be unsafe, the condition shall be immediately reported to the Lead person or Supervisor.  Company vehicles shall be removed from service and not used again until made safe.

- No gas-powered vehicle shall be operated with a leak in fuel system.

- Drivers shall not exceed the authorized or safe speed of 5 mph.  Drivers must keep vehicles under positive control, maintain a safe distance from other vehicles and follow all traffic regulations.  For vehicles traveling in the same direction, a safe distance shall be three vehicle lengths or preferably a time lapse of three seconds, when passing the same point.

- Other vehicles traveling in the same direction shall not be passed at intersections, blind spots or dangerous locations.

- The driver shall slow down and sound the horn at cross aisles and other locations where vision is obstructed.

- Vehicles shall not be driven up to anyone standing in front of a bench or other fixed object where the person could be caught between the vehicle and object.

- Grades shall be ascended or descended slowly.

a.	When ascending or descending grades in excess of 10 percent, loaded forklifts shall be driving with a load 		upgrade.

b.	On all grades, the load and load-engaging means shall be tilted back and raised only as far as necessary to clear 		the road surface.

c.	Motorized hand and hand/rider lifts shall be operated on all grades with the load-engaging means downgrade.

- Forklift forks shall always be carried as low as possible to the ground

- When leaving a vehicle unattended, the power shall be shut off, brakes set, the mast of forklifts brought to the vertical position and the load-engaging means left in the down position.  When left on an incline, the wheels shall be blocked.

		NOTE: A vehicle is considered to be unattended when the vehicle is not within the operator's view or when the operator is more than 25 feet from the vehicle.

- When an operator has dismounted from a forklift which is within his view and he is remaining within 25 feet or the forklift, the load-engaging means shall be lowered, controls neutralized and the brakes set.

- Prior to driving onto trucks or trailers, their flooring shall be checked for breaks or other structural weaknesses.

- Vehicles shall not be driven in and out of trucks or trailers at loading docks until the trucks or trailers are securely blocked and the brakes are set.

- Vehicles shall not be driven within one tire width of the edge of elevated docks, platforms, truck or trailer beds, except in tracks with elevated edges designed for that purpose.

- Trucks and trailers shall not be loaded in excess of their rated capacity.

- A loaded truck or trailer shall not be moved until the load is safe and secured.

- Extreme care shall be used when tilting forklift loads.  The load-engaging means shall not be tilted forward except when picking up a load or when depositing the load onto a rack.  When stacking or tiering, the backward tilt shall be limited to that necessary to stabilize the load.

- When using forklifts, the load-engaging device shall be placed in such a manner that the load will be securely held and supported.

- Extra precautions shall be taken when using forklifts with devices added for securing or handling special loads, and during the operation of these forklifts after the load has been removed.

- If a load being carried obstructs forward view, the driver is required to travel with the load trailing.

- Vehicles may not be driven outside of company property.

Elevating Employees With Lift Trucks

When it is necessary to elevate employees using an industrial truck, the following guidelines shall apply

- The platform shall be of sufficient size (but not less than 24" by 24") to accommodate the employee and the material being elevated.

- The platform floor shall have no spaces or holes, shall be secured to the forks or mast to prevent tipping, slipping or falling and shall have a slip resistant surface.

- The employee and vehicle operator must wear a hard hat.

- There shall be an operator in the control position on the truck while employees are on the elevated platform.

- A lift vehicle must never travel with an employee on the work platform other than to make minor movements for final positioning of the platform.